Cover

BANSHEE (Beacon-Assisted Navigation System for Hostile Environment Exploration) is a modular sensing payload that detects and logs RF beacon signals to support navigation and search tasks in GPS-degraded/denied environments. The current scope emphasizes data acquisition and offline analysis: a Raspberry Pi 5 hosts an SDR front end (most likely LimeSDR) to capture I/Q data in the 915 MHz band, while simultaneously recording timestamp, coarse location (GNSS when available), platform heading (magnetometer/IMU), and system health to an external SSD. Subsystems include: Power/Protection, SDR/RF Front End (LNA/filters/SDR), Compute/Storage (Pi 5 + SSD), PNT/Attitude (GNSS + magnetometer/IMU), and Operator I/O (status display/LEDs and control switch). Data is stored as chunked capture files with sidecar metadata for easy post-mission ingestion. The architecture favors field serviceability (swappable antennas/filters, COTS compute/storage) and deterministic logging (monotonic clock discipline, file rollovers) so teams can rapidly iterate signal-processing pipelines offboard. While autonomy and real-time geolocation are out-of-scope for this semester, the interfaces, timing, and metadata schemas are designed to be forward-compatible with future onboard direction-finding and closed-loop guidance.

Why It’s Important

In disaster response, contested EM environments, or subterranean sites, GPS may be unreliable. BANSHEE gives teams a ground-truth RF log with synchronized context (time, heading, location) so analysts can localize beacons, evaluate link budgets, and plan subsequent sorties. The same data backbone later supports real-time DF and autonomy. So, shipping a robust logger first de-risks the hardest part: clean, contextual data.

Requirements

User Requirement(s)

- USR-1 (Ubiquitous): Continuous RF Capture

The user shall be able to record RF I/Q centered at a user-selected frequency in the 902–928 MHz band, and store captures with time, heading, and location metadata for post-mission analysis for 10 minutes without interruption. ￼

- USR – 2 (Ubiquitous): Operator Awareness And Control

The user shall receive clear visual or display feedback of system state during 		operation (Idle, Arming, Recording, Fault, Safe-Stop), with responsive indication of 	state changes.

- USR-3 (State-driven): Reliable Data Storage And Timestamping

While GNSS is unavailable, the system shall continue recording RF and heading, flagging location as “unknown” without halting capture. The user shall be able to save recorded data to external storage without data loss or corruption during nominal operation. The user shall be able to associate each recorded dataset with accurate time, heading, and (when available) location information for post-mission correlation.

- USR-4 (Unwanted behavior): Robust Field Operation

If input power falls below the brownout threshold, then the system shall safely stop capture, close files, and persist in metadata integrity.

System Requirement(s)

- SYS-1: The system shall record complex I/Q at ≥ 2.0 MS/s (±5%) with 12-bit or higher effective resolution, tunable within 902–928 MHz, for ≥ 10 minutes of continuous capture.

- SYS-2: The system shall timestamp each file start and chunk (≤10 s) with monotonic time and wall-clock time, and log heading (≥1 Hz) and location (≥1 Hz when available).

- SYS-3: The system shall write data and metadata to an external SSD at an average sustained throughput of ≥ 20 MB/s (headroom for 2 MS/s I/Q + metadata), with no data loss events over a 10-minute run.

- SYS-4: The system shall provide operator feedback (LED or display) for capture state: Idle, Arming, Recording, Fault, Safe-Stop (response ≤ 0.5 s to state change).

- SYS-5: The system shall operate from 10–16 V DC input and tolerate brownout down to 9 V long enough to perform a safe stop (<2 s).

Architectural Design (subsystems + one interface)

Subsystems

- PWR: Power & Protection (DC-in, DC-DC, fuse, brownout detect)

- RF: Antenna/LNA/Filter → SDR

- CMP: Compute & Storage (Raspberry Pi 5 + SSD)

- PNT: GNSS + Magnetometer/IMU

- IO: Operator I/O (LEDs/status display, record switch)

Primary Interface Requirement

- IF-A (CMP <-> SDR, USB 3.0): The interface shall sustain ≥ 40 MB/s bulk transfers with <1% packet retries over 10 min capture, and expose SDR controls (tune, gain, sample-rate) via USB control endpoints.

Subsystem Requirement(s)

- RF-1 (RF Subsystem): When tuned to 902–928 MHz, the RF chain shall provide a strong link margin > 20dBm to mitigate gain robbing. Additionally, the RF front end shall lower the noise figure by –3dB +- 0.5dB to enhance signal clarity

- CMP-1 (Compute/Storage): The compute node shall maintain CPU <85% and disk writes latency p95 < 20 ms during 2 MS/s capture.

- PNT-1 (PNT): The PNT stack shall output heading at ≥1 Hz with ±5° RMS and location at ≥1 Hz with horizontal uncertainty estimate when GNSS is available.

- IO-1 (I/O): The front-panel Record control shall initiate capture within 0.5 s and emit distinct indicators for Recording vs Fault.

Tests

User Requirements Tests

USR1: Continuous RF Capture

Requirement:
The user shall be able to record RF I/Q centered at a user-selected frequency in the 902–928 MHz band, and store captures with time, heading, and location metadata for post-mission analysis for 10 minutes without interruption.

Test Procedure:

- Setup:

  - Connect RF signal generator to system input.

  - Power system from nominal 12 V DC.

  - Attach external SSD.

  - Connect GNSS and heading sensor (if available).

- Operation:

  - From the user interface, select a center frequency (e.g., 915 MHz).

  - Begin a capture session.

  - Observe that the system enters Recording state and logs start time.

  - Allow continuous operation for ≥ 10 minutes.

- Post-Run Validation:

  - Confirm capture file(s) exist and cover full 10 min span.

  - Open metadata and verify:

    - Center frequency matches user input.

    - Wall-clock and monotonic increasing timestamps present.

    - Heading and location present at ≥ 1 Hz.

  - Plot I/Q continuity to confirm no sample gaps.

Pass/Fail Criteria

- Recording duration ≥ 10 minutes continuous with no interruptions.

- Frequency, timestamp, and metadata are correctly logged.

- No missing or corrupted data.

- Metadata retrievable and readable in post-mission analysis software.

USR2: Operator Awareness And Control

Requirement:
The user shall receive clear visual or display feedback of system state during operation (Idle, Arming, Recording, Fault, Safe-Stop), with responsive indication of state changes.

Test Procedure:

- Setup: Power on the system and observe the default state.

- Operation: Sequentially command transitions through:

  - Idle → Arming → Recording → Safe-Stop → Idle.

- Fault Simulation:

  - Disconnect the external SSD or induce an RF front-end error to trigger Fault state.

- Observation:

  - Observe LED or display indications for each state.

  - Measure response latency with a stopwatch or logic analyzer.

Pass/Fail Criteria

- Each state has a distinct, unambiguous indicator (LED color/pattern or display message).

- Indicator updates ≤ 0.5 s after state change command or detected event.

- Fault indication persists until the user acknowledges or resets the system.

USR3: Reliable Data Storage And Timestamping

Requirement:
While GNSS is unavailable, the system shall continue recording RF and heading, flagging location as “unknown” without halting capture. The user shall be able to save recorded data to external storage without data loss or corruption during nominal operation. The user shall be able to associate each recorded dataset with accurate time, heading, and (when available) location information for post-mission correlation.

Test Procedure:

- Setup:

  - Operate system with GNSS and heading sensors active.

  - Verify normal capture operation.

- GNSS Loss Simulation:

  - Disconnect or block GNSS antenna mid-capture.

  - Maintain heading sensor connection.

- Observation:

  - Confirm capture continues uninterrupted.

  - Observe system logs or metadata flagging location as “unknown” or “invalid.”

  - After ~10 s, reconnect GNSS and ensure location resumes updating.

- Post-Run Validation:

  - Inspect metadata to verify continuous timestamps and headings.

  - Verify no dropped samples or data loss during GNSS outage.

Pass/Fail Criteria

- Capture continues seamlessly when GNSS data is unavailable.

- Location fields marked “unknown” while GPS lost; resumed after recovery.

- File integrity maintained; no interruptions or data corruption.

- Timestamps, headings, and locations correlate correctly in playback.

USR4: Robust Field Operation

Requirement:
If input power falls below the brownout threshold, then the system shall safely stop capture, close files, and persist in metadata integrity.

Test Procedure:

- Setup:

  - Power system from programmable DC supply.

  - Begin a recording at nominal 12 V.

- Brownout Simulation:

  - Gradually reduce voltage to 9 V while observing system behavior.

- Observation:

  - Verify system enters Safe-Stop or equivalent mode automatically.

  - Confirm active capture is terminated gracefully (no abrupt file truncation).

  - Measure time from voltage crossing threshold to stop confirmation.

- Post-Run Validation:

  - Verify recorded data files can be opened and metadata complete (timestamps, stop reason, file closed).

  - Restore voltage to nominal and confirm system recovery.

Pass/Fail Criteria

- Safe-Stop initiated within 2 s of brownout threshold crossing.

- No corrupted or partial files.

- Metadata integrity is preserved (stop reason logged).

- System returns to Idle after recovery without manual reboot.

System Requirements Tests

SYS1: RF CAPTURE RATE, RESOLUTION, AND DURATION

Requirement:
The system shall record complex I/Q data at ≥ 2.0 MS/s (± 5 %) with ≥ 12-bit effective resolution, tunable within 902–928 MHz, for ≥ 10 minutes of continuous capture.

Test Procedure:

- Connect RF generator to system RF input.

- Set RF generator to a known tone at 915 MHz, 0 dBm.

- Configure system for 2.0 MS/s sampling, 915 MHz center frequency.

- Start recording for 10 minutes.

- Record system logs, file size, and any errors.

- Post-process captured data:

  - Verify sample rate via FFT tone spacing and metadata (± 5 %).

  - Compute 12-bit resolution using SNR from the captured tone.

  - Confirm continuous capture duration ≥ 600 s (no gaps).

Pass/Fail Criteria:

- Sample rate: 1.9–2.1 MS/s

- Number of bits ≥ 12 bits (≈ SNR ≥ 74 dB)

- Capture duration ≥ 600 s without dropped samples or discontinuities.

SYS2: TIMESTAMP, HEADING, AND LOCATION LOGGING

Requirement:
The system shall timestamp each file start and chunk (≤ 10 s) with monotonic time and wall-clock time, and log heading (≥ 1 Hz) and location (≥ 1 Hz when available).

Test Procedure:

- Start a 10-minute recording with GPS connected.

- Inspect metadata or log file for:

  - Start time and chunk timestamps.

  - Monotonic increasing time and wall-clock.

  - Heading and location entries.

- Verify timestamps increment every ≤ 10 s of data.

- Move or rotate the system to produce changing heading/location (or simulate changes).

- Cross-check timestamps against real-time clock and GPS data.

Pass/Fail Criteria:

- Each data chunk (≤ 10 s) includes both wall-clock and monotonic timestamps.

- Timestamps are strictly increasing (no backward jumps).

- Heading and location logged ≥ 1 Hz while available, consistent with sensor motion.

SYS3: CONTINUOUS WRITE AND DATA INTEGRITY

Requirement:
The system shall write data and metadata to an external SSD at ≥ 20 MB/s sustained throughput with no data loss over a 10-minute run.

Test Procedure:

- Attach the designated external SSD.

- Start 10-minute capture at 2 MS/s.

- Measure average write throughput using system telemetry or OS counters.

- Compare total data written vs. expected data volume (2 MS/s × 4 bytes/sample × 600 s ≈ 4.8 GB).

- Verify no error logs, buffer overruns, or missing samples.

- Hash check: compute MD5 or CRC of file(s) before/after transfer.

Pass/Fail Criteria:

- Average sustained throughput ≥ 20 MB/s.

- No data loss, underruns, or write errors.

- File integrity verified by consistent hashes and sample continuity.

SYS4: OPERATOR FEEDBACK

The system shall provide operator feedback (LED or display) for capture states: Idle, Arming, Recording, Fault, Safe-Stop, with ≤ 0.5 s response to state change.

Test Procedure:

- Power on system and observe Idle indication.

- Command transitions through:

  - Idle → Arming → Recording → Safe-Stop → Idle.

- Intentionally trigger a Fault (e.g., disconnect SSD or RF input) and verify Fault indication.

- Use oscilloscope or logic analyzer on LED control lines to measure latency from command or event to LED/display update.

Pass/Fail Criteria:

- Each state displays distinct, documented indication (LED color, pattern, or display text).

- Transition latency ≤ 0.5 s.

- Fault and Safe-Stop correctly latched and cleared only after reset or acknowledgement.

SYS5: POWER INPUT RANGE AND BROWNOUT HANDLING

Requirement:
The system shall operate from 10–16 V DC input and tolerate brownout down to 9 V long enough to perform a safe stop (< 2 s).

Test Procedure:

- Power system at nominal 12 V and verify normal operation.

- Sweep supply voltage slowly from 10 V to 16 V; monitor current draw and functionality.

- During a recording session, step input voltage to 9 V for 3 s.

- Observe behavior:

  - Verify Safe-Stop sequence initiates within 2 s.

  - Confirm file closure and integrity after brownout.

  - Verify the system recovers cleanly once the voltage is restored.

Pass/Fail Criteria:

- Fully operational between 10–16 V DC.

- On drop to 9 V, Safe-Stop triggered within 2 s.

- No file corruption or data loss.

- Recovery is possible without manual intervention beyond power restore.

Subsystem Requirements Test

RF Subsystem — RF-1 Verification Test

Requirement (RF-1): 
When tuned to 902–928 MHz, the RF chain shall provide strong link margin (> 20 dB stated) and reduce effective noise figure by ~3 dB (±0.5 dB) relative to the SDR-only path.

Equipment: 
RF signal generator (902–928 MHz), step attenuator (≥ 60 dB), spectrum analyzer or SDR in calibrated power mode, calibrated noise source (ENR known) + power meter (for Y-factor) or SDR-based NF routine, RF switches to select “Bypass” (SDR only) vs “Chain” (Antenna/LNA/Filter→SDR), cables, 50 Ω terminations.

Setup:

- Build two paths: Bypass (generator → SDR) and Chain (generator → LNA/Filter → SDR).

- Calibrate generator output at DUT input reference plane. Disable AGC; fix SDR gain.

- Set center at 915 MHz (repeat at 905/920/928 MHz for band coverage).

Procedure:

A) Link-margin proxy (sensitivity shift).

- Inject a single tone and step power down in 1–2 dB increments.

- For each path, find the minimum input level that yields target demod metric (e.g., SNR≥10 dB in 10 kHz RBW, or BER≤10⁻³ for a test modulation).

- Compute ΔPmin = Pmin(Bypass) − Pmin(Chain) as the sensitivity improvement (≈ link margin gain).

B) Noise-figure delta (Y-factor).

- For each path, connect the calibrated noise source; capture “hot” and “cold” power; compute NF.

- Compute ΔNF = NF(Bypass) − NF(Chain).

Measurements & Data: 
Record Pmin per frequency & path; record NF per path (and ENR used).

Pass/Fail Criteria:

- Sensitivity improvement (ΔPmin) ≥ 20 dB at 915 MHz and ≥ 18 dB at band edges (allow 2 dB roll-off).

- ΔNF = 3.0 dB ± 0.5 dB improvement vs Bypass at 915 MHz (≥ 2.5 dB at edges).

- No compression: output spectrum shows <1 dB AM-to-PM/SNR degradation at nominal levels.

Compute/Storage — CMP-1 Verification Test

Requirement (CMP-1):

CPU utilization < 85% and p95 disk write latency < 20 ms during 2 MS/s capture.

Equipment:

System under test (Pi 5 + SSD), RF source (or RF replay) to produce a realistic 2 MS/s stream, OS telemetry tools (pidstat, mpstat, iostat, perf), application log with write-latency telemetry (or fsync timing hooks).

Setup:

- Configure capture at 2.0 MS/s, 915 MHz, typical gain, chunk size ≤ 10 s.

- Ensure SSD is the active target filesystem with sufficient free space.

Procedure:

- Start a 12-minute capture (covers the 10-minute spec plus buffer).

- Log CPU (total and per-process) at 1 Hz, and disk write latency distribution (per chunk and per write).

- If available, enable application-level p50/p95/p99 write latency histograms.

- On minute 6, introduce moderate background load (e.g., 10–15% CPU via stress-ng) to emulate UI/telemetry overhead; keep I/O load off the SSD.

Measurements & Data:

- Average/peak CPU (%) over the run; p95 write latency (ms); underrun/overrun counters.

Pass/Fail:

- Mean CPU ≤ 75%, peak CPU < 85%.

- p95 write latency < 20 ms; no buffer underruns; no dropped samples across 10 min window.

- Total bytes written within ±3% of expected (sample_rate × 4 B × duration + metadata).

PNT/Attitude — PNT-1 Verification Test

Requirement (PNT-1):

Heading ≥ 1 Hz with ±5° RMS and location ≥ 1 Hz with horizontal uncertainty when GNSS is available.

Equipment:

Turntable with angle markings or laser/protractor jig, GNSS antenna with sky view, second GNSS/IMU reference (optional), data logger.

Setup:

- Place system on a level surface; align chassis “nose” with a 0° reference.

- Attach GNSS with clear sky; wait for fix; start a 15-minute session logging heading & location at ≥1 Hz (and uncertainty).

Procedure:

Heading:

- Rotate the platform by known steps: 0°→45°→90°→135°→180°→225°→270°→315°→0°, pausing ≥10 s at each stop.

- Record reported heading; compute error at each stop; then RMS across all stops.

Location:

- Keep system stationary for 5 min; log latitude/longitude/uncertainty.

- Walk a simple 50–100 m rectangle (four corners), pausing 10 s at corners.

- Compare logged track to known geometry (or reference GNSS).

Measurements & Data:

Heading error per stop and RMS; location update interval hist (Hz); reported horizontal uncertainty vs observed scatter (CEP).

Pass/Fail:

- Heading output rate ≥ 1 Hz; RMS error ≤ 5°.

- Location output rate ≥ 1 Hz when GNSS has fix; reported uncertainty present on each sample.

- Stationary scatter consistent with reported uncertainty (e.g., 50–70% of points inside stated 1-σ/CEP radius).

Operator I/O — IO-1 Verification Test

Requirement (IO-1):

Front-panel Record control shall initiate capture within 0.5 s and emit distinct indicators for Recording vs Fault.

Equipment:

Logic analyzer or scope (monitor GPIO for “record armed/active”), photodiode on LED/display backlight (optional), external SSD to provoke/remove faults, stopwatch.

Setup:

- Expose a digital “capture_active” signal (GPIO/log event).

- Position photodiode to read LED/display transitions (optional but ideal for latency).

Procedure:

- From Idle, press Record once. Measure latency from button edge (or GPIO) to capture_active=1 and first LED/display transition to “Recording.”

- While recording, unplug SSD to force Fault. Measure latency from fault event (kernel/dmesg or app detect) to “Fault” indicator.

- Restore SSD; acknowledge/reset; verify return to Idle.

Measurements & Data:

Latency histograms over 10 trials; indicator patterns captured (photo/video logs acceptable).

Pass/Fail:

- Start latency ≤ 0.5 s (p95) from button actuation to capture active and visible “Recording” indication.

- Fault indication is distinct and appears ≤ 0.5 s after fault detect; persists until user acknowledgment.

- Clear separation of Recording vs Fault indications per UI spec.